INTENT: To construct 6 two-family structures (12 units in total) with the single-family dwelling to remain on the property. In addition to the Copper Drive Project under construction.

UNITS PROPOSED: 12, plus existing (13 in total)

AGENDA ITEM #: 3

ACREAGE:

Existing = 0.47± ac.

Post Lot Merger = 103,666± sq. ft.

ZONING DISTRICT: G

EXISTING LAND USE: Single-family residence

PROPOSED LAND USE: Add 12 residential units

SURROUNDING LAND USE: Residential, Willand Pond

ZBA ACTION: None

PERMITS REQUIRED:

- EPA NOI

- NHDES Sewer Discharge Permit

- CUPs for lot coverage, frontage build out, front build to line.

WAIVERS REQUESTED:

- Drainage pipe

ATTENDANCE:

Donna Benton, Planning

Paul Crouser, Zoning

Marn Speidel, Police

Jim Maxfield, Fire/Inspection Services

Ken Mavrogeorge, Engineering

James Burdin, Business Development

Others: Kevin Poulin

Robert Baldwin

Erin Bassegio, Planning

PLAN REVIEW:

Planning:

- Impact fees

- Water and sewer investment fees

- CUPs for lot coverage, frontage build out, front build to line. Only 3 non-enviro CUPs needed

- Sidewalk contribution

- Revise plans:

  - Owner’s signatures on the final plans

  - Professional stamps on the final plans

  - Confirm lighting meets regulations

- Lot merger as a COA

- Active and passive rec fee as a COA

- Contribute towards sewer modeling

- HOA documents to be reviewed

- Development Agreement needed

- Section 157-31.F. Block size requires no less than 500’

Engineering:

- General Comment:

  - It is strongly recommended that the Applicant submit a detailed breakdown of proposed HOA costs for annual maintenance and replacement of private infrastructure including backup as to how the fees are estimated. Engineering supports this as a condition of approval.

  - Remove ADA detectable warnings at driveway crossings on New Rochester Rd.

  - Coordinate painting of crosswalks in New Rochester Rd with CS.

- Sheet 6 of 28:

  - Recommend removing or reducing the curb cut for units 16 & 17 on New Rochester Rd.

  - Will an underdrain be provided to ensure stormwater does not exceed the reservoir course?

- Sheet 7 of 28:

  - Applicant may want to consider providing one fire service to the building in lieu of the proposed two.

  - Applicant to clarify why the water main is shown as bold. Does this imply it is insulated?

- Drainage Analysis:

  - Total Areas analyzed do not match. Please correct.

  - Alteration of Terrain Permit does not appear on the Cover sheet of the plan set or mentioned in the Drainage analysis. Please confirm that an application is required for

Copper and Ember Dr given that the disturbance collectively exceeds the 100,000 sf threshold.

Police:

- Previous comments addressed.

- 14’ lamp near rear of property.

Fire/Inspection Services:

- Sheet 6 note 36 revise Chapter 81 to reference the City of Dover Code.

- Applicant to cite to NH Building/Fire code, when applicable, due to state legislative amendments.

Business Development:

- Previous comments addressed. No new comments.

Planning Board:

- N/A

Next Steps: Applicant to revise and resubmit for January 23, 2024 Planning Board.

Adjournment:

J.Maxfield  moved to adjourn at 12:37 pm. Seconded by: M. Speidel. Vote: U/A.